K Kenneth Prathap

241901048

EXPT: 10 DESIGN A SIMPLE TOPOLOGY AND CONFIGURE WITH ONE ROUTER, TWO SWITCHES AND PCS USING CISCO PACKET TRACER

Aim:

To design and configure a simple network topology using one router, two switches, and PCs in Cisco Packet Tracer and verify successful communication between networks.

Introduction:

In networking, routers are used to connect multiple networks. 
Switches connect devices within a single LAN. 
In this experiment, two LANs (connected by switches) are linked using a router. 
Each LAN will have its own IP network, and the router will route packets between them.

Algorithm:

- Start Cisco Packet Tracer.

- Select and place devices:

- 1 Router (e.g., Cisco 2911)

- 2 Switches (e.g., 2960)

- 4 PCs

- Connect the devices using Copper Straight-Through cables:

- PC0 → Switch0 (F0/1)

- PC1 → Switch0 (F0/2)

- PC2 → Switch1 (F0/1)

- PC3 → Switch1 (F0/2)

- Switch0 → Router (G0/0)

- Switch1 → Router (G0/1)

- Assign IP addresses to PCs:

- LAN1 → 192.168.1.0/24 (PC0, PC1)

- LAN2 → 192.168.2.0/24 (PC2, PC3)

- Configure router interfaces:

- Interface G0/0 → 192.168.1.1 255.255.255.0

- Interface G0/1 → 192.168.2.1 255.255.255.0

K Kenneth Prathap

241901048

- Use no shutdown command to activate interfaces.

- Set Default Gateway on each PC:

- For PCs in LAN1 → 192.168.1.1

- For PCs in LAN2 → 192.168.2.1

- Verify connections:

- Use the ping command from one PC in LAN1 to a PC in LAN2.

- Check for successful replies.

- Stop.

- If packets are successfully received, the topology is working correctly.

Network Topology:

Output:

Result:

A simple network topology using one router, two switches, and multiple PCs was designed and configured successfully in Cisco Packet Tracer. Communication between both networks was verified using the ping command.